V-Slot Bearing Gantry System Project

Pina Cammisano, 6235654

Learning today Leading tomorrow

Department of Computer Engineering Technology

Workshop Measurements

Table of Contents

Introduction	3

Objectives	3

Material Used	3

Procedure	3

Results and Discussion	3

Conclusion	3

References	3

Index	4

# Introduction

The V-Slot Bearing Gantry System is a linear motion system designed to move a gantry plate back and forth along an aluminum extrusion track using a belt-driven mechanism. Powered by a NEMA 17 stepper motor, the system is controlled by an Arduino micro- controller interfaced with a TB6600 stepper motor driver. The gantry’s motion is guided by a V-slot bearing system, ensuring smooth and precise movement. Two limit switches, mounted on 3D-printed components, detect the gantry’s position at the track ends, triggering the motor to reverse direction for continuous motion. The system is powered by a dedicated power supply connected to a wall outlet. This project integrates mechanical design, 3D printing, and embedded systems to create a functional linear actuator suitable for applications such as CNC machines or automated systems.

# Objectives

- Design and assemble a belt-driven gantry system using V-slot bearings and aluminum extrusion

- Implement precise control of a Nema 17 stepper motor using an Arduino and TB6600

- Integrate limit switches to enable automatic direction reversal for continuous gantry motion

- Create 3D-printed components (motor holder and extrusion holder) for secure and aligned assembly

- Develop and test Arduino code to manage motor pulses and switch inputs for reliable operation

# Material Used

- Nema 17 stepper motor

- 3D-printed moor holder

- 3D-printed aluminum extrusion holder

- Aluminum extrusion (V-Slot)

- Gantry plate

- TB6600 Stepper motor driver 4A

- Miniature hinge roller lever micro switch

- 12V SA 60W AC/DC Power Supply

- GT2 pulley (16 teeth)

- GT2 Idler timing puller

- Belt

- Arduino uno

- Wall outlet for power supply

- Wires

- Logbook

# Procedure

- The first thing done for this project was creating all our materials on inventor

- Once all the materials were created, I then placed everything on a wooden board (on inventor) to figure out what was the most logical placement

- Bought all the parts needed

- 3-D printed the holders

- Began screwing all the parts on wooden platform

- Began to wire everything together

- Tested my wiring with a basic code given by the teacher

- Began working on the final code

- Tested everything

# Results and Discussion

The V-slot bearing gantry system successfully achieved continuous back-and-forth motion

along the aluminum extrusion track. The key results and observations are:

• Mechanical Performance: The V-slot bearings provided smooth and stable motion, with the belt maintaining consistent tension. the 3D-printed motor holder caused slight belt slippage.

• Electrical Functionality: The TB6600 driver reliably controlled the NEMA 17 motor, with the Arduino’s 500-microsecond pulse interval ensuring smooth operation. The active-high limit switches (pins 5 and 6) accurately detected the gantry’s endpoints, triggering direction changes.

• Control Logic: The Arduino code (shown below) effectively managed motor direction and pulses. The 50ms debounce delay prevented false triggers from switch noise.

Listing 1: Arduino Code for V-Slot Gantry System

Flowchart: The control logic flowchart, created in Microsoft Visio, includes:

Schematic Diagram:

Figure 3: 3D Model of V-Slot Gantry System:

- Attached to submission

Figure 4: Part Placement of Gantry System

Figure 5: Final V-Slot Gantry System

# Conclusion

The V-slot bearing gantry system successfully achieved continuous linear motion, with the NEMA 17 stepper motor driving the gantry plate along the aluminum extrusion via a belt. The limit switches, configured as normally open (NO) and active-high, reliably triggered direction reversal. In the NO configuration, each switch’s circuit is open when not pressed, outputting LOW (0V) to Arduino pins 5 and 6. When the gantry presses a switch, the circuit closes, pulling the pin to HIGH (5V) via an external pull-up resistor or the TB6600’s 5V supply, as detected by ‘digitalRead(limitSwitch1) == HIGH‘ and ‘digitalRead(limitSwitch2) == HIGH‘. The TB6600’s ENA pin is active-low, enabled setting pin 2 to LOW. This setup simplified the code but required careful wiring to avoid noise.

Switch Schematic:

• NO Configuration: One switch terminal connects to Arduino pin 5 or 6, the other

to ground. When pressed, the switch closes, connecting the pin to 5V (active-high)

via a pull-up resistor.

• Alternative (NC): A normally closed switch would connect the pin to ground when not

pressed and open when pressed.

Lessons Learned:

• Switch Logic: Using active-high NO switches taught me to align hardware and

software for reliable signal detection, with debouncing (50ms delay) critical to avoid

false triggers.

• Mechanical Design: 3D printing required precise tolerances for the motor holder

and belt alignment to prevent slippage.

• Electronics: Proper grounding and pull-up resistor configuration were essential for

stable switch operation.

• Programming: Using ‘micros()‘ for pulse timing ensured smooth motor control

without blocking the loop.

What I Would Do Differently:

• Switch Configuration: I would test normally closed (NC) switches to reduce noise

susceptibility, adjusting the code to use ‘INPUTP U LLU P ‘and check for LOW signals.

• Early Prototype: I would print my parts earlier to address alignment issues sooner.

• Wiring Organization: I would us the proper coloured wires to make my life simpler when debugging.